Smart Agriculture IoT System

An ESP32-based smart agriculture monitoring and control system using ThingSpeak Cloud.

Features

- Monitors soil moisture, rain, temperature, humidity, light intensity, and water-tank level.

- Automatically turns the LED on when light intensity is below 30%.

- Controls a water pump through an L298N motor driver.

- Reads app pump ON/OFF commands from a ThingSpeak control channel.

- Blocks pump operation when the water tank level is 40% or lower.

- Forces the pump OFF when the water level drops below 20%.

- Uploads sensor readings and pump/LED status to a ThingSpeak telemetry channel.

- Prints sensor values and device status to the Serial Monitor.

Hardware Required

- ESP32

- Soil-moisture sensor

- Rain sensor

- DHT11 temperature and humidity sensor

- light sensor module

- water-level sensor

- L298N motor driver

- DC water pump

- LED

- Jumper wires and suitable power supply

- Wi-Fi network with internet access

Pin Connections

| Component | ESP32 pin |
| --- | --- |
| Soil-moisture analog output | GPIO 34 |
| Rain sensor digital output | GPIO 27 |
| DHT11 data | GPIO 4 |
| LDR analog output | GPIO 35 |
| Water-level analog output | GPIO 32 |
| L298N IN1 (pump) | GPIO 25 |
| L298N IN2 (pump) | GPIO 26 |
| LED control | GPIO 33 |

- Battery + to 12v+ in L298N.

- Battery – to GND in L298N.

- from L298N 5V+ to all Sensor VCC and Esp32 Vin using Bread Board.

- from L298N GND to all sensor GND and Esp32 GND using Bread Board.

Software Requirements

Arduino IDE

ThingSpeak account

Creating ThingSpeak Cloud

Open ThingSpeak and sign in or create an account.

Creation of Channel 1:

- Go to Menu  Select Channels  My Channels  New Channel.

- Name  Telemetry Channel.

- Enable Fields 1–8 (tick the Check Boxes).

| ThingSpeak field | Type the Field label |
| --- | --- |
| Field 1 | Soil moisture (%) |
| Field 2 | Rain status |
| Field 3 | Temperature (°C) |
| Field 4 | Humidity (%) |
| Field 5 | Light intensity (%) |
| Field 6 | Water-tank level (%) |
| Field 7 | Pump status |
| Field 8 | LED status |

- Scroll down and Save Channel.

Creation of Channel 2

- Go to Menu  Select Channels  My Channels  New Channel.

- Name  Control Channel.

- Enable Field 1 and Fill the Box as Pump Command.

- Scroll Down and Save Channel.

Filling of API Keys in the App.

- Download the apk file from the Github, and install the apk in the mobile.

- Go to Thingspeak   Menu  My Channel   Telemetry channel  API Keys  Copy the Write API Key and Read API Key and Channel ID and paste in the Esp32 Code in the place of “Enter Telemetry Channel ID” and “Enter Telemetry Channel Write Key”. Paste the Channel ID and Read Key in app settings.

- Go to Thingspeak   Menu  My Channel   Control channel  API Keys  Copy the Write API Key and Read API Key and Channel ID and paste in the Esp32 Code in the place of “Enter Control Channel ID” and “Enter Control Channel Read Key”. Paste the Control Channel ID and Write Key in app settings.

- Save the settings in the App in mobile.

Uploading Code

- Open Arduino IDE and Select ESP32 board and port.

- Install the DHT sensor library by Adafruit from Library Manager.

- Replace the Wi-Fi placeholder with your network name and password.

- Compile and upload the sketch.

- Open Serial Monitor at 115200 baud.

- Check that Wi-Fi connects and sensor readings appear. Verify the readings against the physical sensors before relying on automatic control.

Important Notes

- Replace every placeholder before uploading. Never publish real Wi-Fi passwords or private API keys in a public GitHub repository.

- If credentials were committed publicly, change the Wi-Fi password/API keys and remove the exposed values from the repository history where appropriate.

- The sketch calls client.setInsecure() for HTTPS, which skips certificate verification. This is convenient for some prototypes but is not recommended for production security.

- ThingSpeak normally enforces an update interval on free channels. The sketch uses 15-second telemetry and control intervals; check your account/channel limits before changing them.

- The percentages are estimated from calibrated analog readings, not laboratory measurements.

- Test the pump with the motor driver and a safe power supply, and keep water away from exposed electronics.